Assessing phytotoxicity of trace element-contaminated soils phytomanaged with gentle remediation options at ten European field trials.

# Quintela-Sabarís Celestinoa,b,*, Marchand Liliana, Kidd Petra Sb, Friesl-Hanl Wolfgangc, Puschenreiter Markusd, Kumpiene Juratee, Müller Ingof, Neu Silkef, Janssen Jolieng, Vangronsveld Jacog, Dimitriou Ioannish, Siebielec Grzegorzi, Gałązka Rafałi, Bert Valériej, Herzig Rolfk, Cundy Andrew Bl, Oustrière Nadègea, Kolbas Aliaksandra,m, Galland Williama, Mench Michela

Affiliations:

aBIOGECO, INRA, Univ. Bordeaux, 33615 Pessac, France

bInstituto de Investigaciones Agrobiológicas de Galicia (IIAG), Consejo Superior de Investigaciones Científicas (CSIC), Santiago de Compostela 15706, Spain

cAIT Austrian Institute of Technology GmbH, Energy Department, 3430 Tulln, Austria

dUniversity of Natural Resources and Life Sciences Vienna – BOKU, Department of Forest and Soil Sciences, 3430 Tulln, Austria

eLuleå University of Technology, Waste Science & Technology, SE-97187 Luleå, Sweden

fSaxon State Office for Environment, Agriculture and Geology, Pillnitzer Platz 3, 01326 Dresden Pillnitz, Germany

gHasselt University, Centre for Environmental Sciences, 23 Agoralaan building D, B-3590 Diepenbeek, Belgium

hSwedish University of Agriculture Sciences, Department of Crop Production Ecology, SE-750 07 Uppsala, Sweden

iInstitute of Soil Science and Plant Cultivation - State Research Institute, Czartoryskich 8, 24-100 Pulawy, Poland

jINERIS, Technologies and Sustainable and Clean Processes, Parc Technologique Alata, BP2, 60650 Verneuil en Halatte, France

kPhytotech Foundation (PT-F), and AGB-Bioindikation Umweltbeobachtung und oekologische Planung Quartiergasse, Bern, Switzerland

lOcean and Earth Science, National Oceanography Centre (Southampton), University of Southampton, Southampton, SO14 3ZH, UK

mBrest State University named after A.S. Poushkin, 224016, Brest, Belarus

* Corresponding author: Celestino Quintela-Sabarís, email: tino.quintela.sabaris@gmail.com

Abstract

Gentle remediation options (GRO), i.e. in situ stabilization, (aided) phytoextraction and (aided) phytostabilisation, were implemented at ten European sites contaminated with trace elements (TE) from various anthropogenic sources: mining, atmospheric fallout, landfill leachates, wood preservatives, dredged-sediments, and dumped wastes. To assess the performance of the GRO options, topsoil was collected from each field trial, potted, and cultivated with lettuce (Lactuca sativa L.) for 48 days. Shoot dry weight (DW) yield, photosynthesis efficiency and major element and TE concentrations in the soil pore water and lettuce shoots were measured.

GRO implementation had a limited effect on TE concentrations in the soil pore water, although use of multivariate Co-inertia Analysis revealed a clear amelioration effect in phytomanaged soils. Phytomanagement increased shoot DW yield at all industrial and mine sites, whereas in agricultural soils improvements were produced in one out of five sites. Photosynthesis efficiency was less sensitive than changes in shoot biomass and did not discriminate changes in soil conditions.

Based on lettuce shoot DW yield, compost amendment followed by phytoextraction yielded better results than phytostabilisation; moreover shoot ionome data proved that, depending on initial soil conditions, recurrent compost application may be required to maintain crop production with common shoot nutrient concentrations.

Keywords

Biomass, chlorophyll fluorescence, GREENLAND project, lettuce, phytoextraction, phytostabilisation

1. Introduction

More than two centuries of large-scale industrial and urban development has produced a significant legacy of contaminated land in Europe: around 2.5 million sites are potentially polluted (EEA 2014) and around 342 000 contaminated sites have been identified (Panagos et al. 2013). Among soil contaminants, trace elements (TE) (37%), mineral oil (38%) and polycyclic aromatic hydrocarbons (PAH, 13%) are the most common contaminants of concern (Mench et al. 2010). The United States, Canada, Australia, Asian countries (notably China) and developing countries are also experiencing considerable topsoil contamination, posing risks both to the environment and human health (FAO and ITPS, 2015). The legacy of industrial, urban, and intensive agriculture activities may persist in ecosystems for many years after such activities have ceased (Kostaleros et al. 2015). In addition, soil contaminants may be transferred over time to other environmental compartments, e.g. aquatic ecosystems, or dispersed by wind, and so (if risks from the contamination are identified as having significant potential to cause harm to human health or the site or surrounding ecosystems) it is necessary to set up management programs or other interventions, which may include soil remediation. Conventional technologies for soil remediation are often expensive and destructive (e.g. solidification and stabilization, soil flushing, electrokinetics, chemical reduction/oxidation, soil washing, incineration, vitrification, excavation/ retrieval, landfill and disposal, etc.) (Meuser 2013, Yao et al. 2013). A new paradigm supported by various scientific networks (e.g., International Phytotechnology Society, Society of Environmental Toxicology and Chemistry, SETAC; Society for Ecological Restoration, SER) and based on sustainable and low-cost approaches with particular focus on phytotechnologies and ecological restoration tools is emerging however for contaminated site and wider brownfield (phyto)management (Cundy et al. 2013, Kidd et al. 2015, Kaputschka et al. 2015, Cundy et al. 2016). These approaches, termed Gentle soil Remediation Options (GRO), aim to disrupt contaminant linkages (Defra, 2012) either: by controlling the contaminant source (e.g. extracting contaminants from the root zone and subsurface); by managing contaminant pathway(s) (e.g. preventing migration of contaminants); by protecting the receptor(s) (e.g. planning or institutional controls to avoid sensitive land uses) or by some amalgamation of these components (Cundy et al. 2013). The main GRO benefits are that they are less invasive, more restorative to soil structure, ecological functions and related ecosystem services, and their ecological footprint is much lower than many conventional soil remediation techniques (Kumpiene et al. 2014). In addition, as well as providing effective risk management, their use can generate a range of wider environmental, economic and societal benefits (Cundy et al. 2016). Within the term GRO various techniques are included, from in situ contaminant stabilisation (“inactivation”) to phytomanagement (plant-based) options. Phytomanagement can be directed to different objectives, from TE removal from soil (phytoextraction) to their stabilisation in contaminated soils (phytostabilisation), and includes the use of amendments to modify TE mobility or the inclusion of soil microorganisms in the system (bioaugmentation) (Kidd et al. 2009, 2015; Mench et al. 2010).

One of the critical points of the implementation of GRO is to assess the initial and the residual risk after GRO application, notably using relevant standard ecotoxicological batteries as proxies of metal(loid) availability and soil ecotoxicity (Kumpiene et al. 2014). Here, the residual phytotoxicity of phytomanaged soils from ten established European field trials testing three GRO was assessed using lettuce (Lactuca sativa L.) as biomonitor. The sites were contaminated with trace elements (TE) from various anthropogenic sources and the effects of different implemented GRO were assessed on TE concentrations in the soil pore water (SPW) and the (1M NH4NO3)-extractable soil fraction (environmental TE availability), shoot TE concentrations (environmental bioavailability), and shoot DW yield and photosystem II activity (toxicological bioavailability).

2. Material and Methods

2.1. Sites and soils

Contaminated soils were collected in ten field trials where GROs were implemented as part of the EU Greenland project (8 European countries). These sites are described in Kidd et al. (2015), Kumpiene et al. (2014) and Puschenreiter et al. (2014). Information on the sites and treatments are summarized in Table 1, and presented in more detail as supplementary materials S1, S2 and S7. Phytoextraction (i.e. soil TE removal by accumulating them in harvestable biomass of plants) was implemented in five sites, whereas in three sites phytostabilisation (i.e. the reduction in the bioavailability of pollutants by immobilisation in root systems and / or living or dead biomass in the rhizosphere soil) and phytoexclusion (i.e. the implementation of excluder plants not accumulating contaminants in the harvested biomass nor having significant influence on the labile pool of soil contaminants) were applied. In two sites (BIOGECO and Touro) both approaches were used. An uncontaminated soil (Eutric Gleysol, World Reference Base for soil resources) from a kitchen garden (Kolbas et al., 2011) was used as a control soil (CON, table S2).

At each site, a random sampling within a systematic rectangular grid area was applied (Kumpiene et al. 2014). Topsoils (0 - 20 cm) were collected using an unpainted spade in autumn 2013. Three independent composite samples (each made of three subsamples, total weight 2 kg DW) were taken from each site and soil treatment. Soil samples were homogenised and sieved to <4 mm prior to shipping and the setup of the pot experiments. Ammonium nitrate (NH4NO3)-extractable TE concentrations were determined by the standard procedure (DIN ISO, 19730:2008 (E)). Air-dried, sieved to < 2 mm, soil was extracted with 1M NH4NO3 solution at a liquid-to-solid ratio (L/S) of 2.5 L kg−1 for 120 min using an end-over-end shaker at room temperature and filtered.

Table 1: Overview of sites. Sites are ordered according to the GRO applied. Country abbreviation is indicated within brackets next to site name. In the last column the nomenclature corresponding to each treatment are indicated.

| GRO/Site (country) | Main contaminant | Treatments | Labels |
| --- | --- | --- | --- |
| (Aided) Phytoextraction |  |  |  |
| Bettwiesen (CH) | Cd, Zn, Pb | Untreated agricultural soil on old landfill | UNT |
|  |  | TE removal using tobacco lines | PE |
| Freiberg-Halsbrücke (DE) | As, Cd, Pb | Untreated polluted agricultural soil | UNT |
|  |  | TE removal using Salix clone TORA | PE |
| Högbytorp (SE) | (Cd), Cr, Zn | Agricultural soil next to a landfill | UNT |
|  |  | Agricultural plots irrigated with landfill leachates and TE removal using Salix clone TORA | PE |
| Ineris (Creil, Phytoagglo) (FR) | Cd, Zn | Urban brownfield, TE removal using Salix viminalis L. | PE |
| Lommel (BE) | Cd, Zn, Pb | Untreated agricultural soil contaminated by fallout | UNT |
|  |  | TE removal using Salix clone TORA | PE |
| Biogeco (St-Médard d’Eyrans) (FR) | Cu | Untreated soil at a wood preservation site | UNT |
|  |  | Compost (1 or 2 times) and dolomitic limestone and TE removal by crop rotation tobacco-sunflower | PE1 |
|  |  |  | PE2 |
| Touro (ES) | Cu | Untreated soil on mine tailings | UNT |
|  |  | Compost amendment and no planting | UNT+OM |
|  |  | Compost added, TE removal using tobacco lines | PE |
| In situ immobilisation/ phytoexclusion and phytostabilisation |  |  |  |
| Arnoldstein (AT) | As, Cd, Zn, Pb | Untreated polluted agricultural soil | UNT |
|  |  | Soil amendment and maize-barley cultivation | PS |
| Ineris (Fresnes-sur-Escaut, Phytosed Ec 1) (FR) | Cd, Pb, Zn, Cu | Soil amendment (basic slag: Optiscor) and planting of Deschampsia cespitosa L. with two willow clones (Inger, Tordis) | PS |
| Piekary (PL) | Zn, Cd, Pb | Untreated soil on smelter slags and dumped wastes | UNT |
|  |  | Biosolid (low conc.) and grass cultivation | PS1 |
|  |  | Biosolid (high conc.) and grass cultivation | PS2 |
| Biogeco (St-Médard d’Eyrans) (FR) | Cu | Untreated soil at wood preservative site | UNT |
|  |  | Compost + iron grit and cultivation of grassy and woody species* | PS |
| Touro (ES) | Cu | Untreated mine tailing soil | UNT |
|  |  | Compost amendment and no planting | UNT+OM |
|  |  | Compost added, planting of Salix caprea L. Mauerbach (BOKU 01 AT-004), S. viminalis L.** and Populus nigra L.§ | PS |

*Populus nigra L., Cytisus striatus (Hill) Rothm., Agrostis capillaris L., A. gigantea Roth., ** native population from the Touro site, § native population from the Biogeco site

2.2. Plant test

The Plantox test using lettuce (Lactuca sativa L.) was performed following the standard procedure (adapted from ISO, 17126:2005, Kumpiene et al. 2014). Briefly, in January 2014 (week 2), soil samples (3 replicates by site and treatment, including the control soil) were sieved at 4 mm, potted in 0.65 L plastic pots (around 1 kg soil DW per pot) and placed in a greenhouse (INRA,Villenave d'Ornon, France). One rhizon MOM moisture sampler (Eijkelkamp, The Netherlands) was inserted with a 45º angle into each potted soil. Soils were fertilized with 100 mL of a modified Hoagland n°2 solution to avoid nutrient deficiencies, rehydrated up to their water holding capacity (WHC) by capillarity, and then maintained between 60% and 80% of WHC by daily manual irrigation with deionised water (no leaching). Soil conditions were left to stabilize for 3 weeks before the experiments began.

The day prior to experimental commencement (week 5), 20 mL of soil pore water (SPW) were collected from each pot using the rhizon sampler. The pH (Hanna instruments, pH 210, combined electrode Ag/AgCl, USA) and electrical conductivity (EC) (WTW Multiline P4 meter, Germany) were measured in each SPW sample. Afterwards, SPW samples were acidified with a drop of HNO3 and kept at 4ºC prior to element analysis by either ICP-AES (Liberty 200, Varian Inc., Australia) or ICP-MS (X Series 200, Thermo Elemental, England) at USRAVE, INRA (Villenave-d'Ornon, France) using standard methods (Afnor ISO 11885, 2009).

In February 2014 (week 6), one lettuce plantlet (cv. Novappia, 5–6th leaf stage) was transplanted into each pot in triplicates for all treatments. Pots were placed in a greenhouse under natural light conditions and day/night temperatures of 18 ± 6 /11 ± 2 ºC, positions being randomised each week. Pots were daily and manually irrigated with deionised water to keep soil humidity between 60% and 80% of WHC. Lettuce plants were harvested after a 48-d growth period and shoot fresh weight (FW) yields were determined. Shoots were then thoroughly washed in tap water followed by deionised water, put in paper bags and oven-dried at 50°C until constant weight (during 48h). Shoot dry weight (DW) yield was determined prior to grinding (<1 mm). For each shoot sample, a weighed aliquot (0.1 g DW) was digested in a 2:1 concentrated HNO3:HCl mixture (3 mL) on a hot plate at 130°C and the volume was made up to 10 mL. The ionome in digested samples was measured by inductively coupled plasma optical emission spectrometry (ICP-OES) (Vista Pro; Varian Inc., Australia) at the IIAG-CSIC laboratory (Santiago de Compostela, Spain) and expressed in mg kg-1 DW plant material.

Photosynthesis efficiency, as estimated by chlorophyll fluorescence, was measured in vivo on three fully expanded leaves of each plant the day prior to harvesting, using a pulse-amplitude modulated fluorometer (Mini-Pam, Walz, Effeltrich, Germany). The maximum quantum yield of photosystem II (PSII) was assessed from the ratio Fv ⁄Fm = (Fm - F0) ⁄Fm (Bolhàr-Nordenkampf et al. 1989), where F0 and Fm are defined as minimal and maximal fluorescence yields of a dark-adapted sample, with all PSII reaction centers fully open. This parameter was measured with dark-adapted plants to ensure that all their PSII reaction centers were open. The maximum quantum yield estimates the efficiency of excitation energy capture by open PSII reaction centers (Butler and Kitajima 1975).

2.3. Data analysis

According to results of SPW analyses (see below) contaminated sites were split in two groups: soil Cu contamination and Cu exposure were obviously mainly involved for the Touro and Biogeco sites, whereas labile pools of Zn and Cd (and Pb in a lesser extent) were pivotal for the other sites. This was an outcome of Kumpiene et al (2014) (the whole dataset was tested but data were better discriminated and explained by considering two groups based on dominant TE exposure). Statistical analyses were applied separately to each group of sites.

Effects of GRO treatments on soluble TE concentrations, shoot parameters (biomass -FW and DW-, maximum efficiency of PSII and shoot ionome) and shoot TE removals (computed as the product of shoot DW biomass by shoot element concentration) were assessed for each site. For this, Student T-test and one-way ANOVAs were respectively used when two or more treatments were applied per site. One way ANOVAs were completed with Tukey post-hoc tests to assess multi-comparison of means between treatments. Data were log-transformed when necessary to meet ANOVA requirements of normality and variance homogeneity. Correlations between TE concentrations in the soil pore water and the (1M NH4NO3) extractable fraction were assessed by a non-parametric Spearman correlation index. ANOVA, student T tests, Tukey post hoc tests, and Spearman index analyses were performed using the SPSS package (v. 15, SPSS Inc., Chicago, IL, USA).

Chemical analyses of SPW and shoots resulted in two multivariate data sets (for both site groups), one consisting of pH and soluble TE concentrations and the other consisting of shoot ionome data. A Co-inertia Analysis (CoIA, Doledec and Chessel 1994) was performed to check for co-structure between the two data sets. In brief, CoIA is a two-table ordination method that finds a common space onto which the samples and the variables of the data sets can be projected and compared (Legendre and Legendre 2012). In this common space, the covariance (and also the correlation and standard deviation) between the two new sets of projected scores is maximal. Prior to CoIA, each data set was processed by Principal Component Analysis (PCA). Moreover, the coefficient of correlation between the two datasets (RV coefficient) was computed, and its significance evaluated by a permutation test. The Ineris field trials were excluded from the CoIA analysis due to the lack of untreated soil samples for comparison. The PCA and CoIA analyses were performed with different functionalities of the ade4 package (Chessel et al. 2004; Dray et al. 2007) using R software (version 2.12.0, R Foundation for Statistical Computing, Vienna, Austria).

3. Results

3.1. Shoot DW yield and chlorophyll fluorescence

Lettuce plants growing on untreated soils had, in general, lower shoot DW yields than those grown in the control soil, except in the case of Bettwiesen (the UNT plants had a higher shoot biomass than the controls) (Fig. 1, Table S3). In four out of the ten sites, shoot DW yield differed between plants growing on untreated and phytomanaged soils. Lettuces growing on the Lommel phytomanaged soil had an intermediate shoot biomass between the untreated and control soils, whereas the Piekary phytomanaged soils yielded higher shoot biomasses than the untreated soil (Fig. 1). At both Cu-contaminated sites (Biogeco and Touro), phytoextraction had a slightly better effect on shoot biomass than phytostabilisation: lettuces grown on soils managed by the PE2 (Biogeco) and PE (Touro) treatments had similar biomass values as plants grown on the control soil, whereas lettuces from PS treatments had intermediate values (Touro) or lower biomass (Biogeco) than controls (Fig. 1). For the other sites, plants grown on phytomanaged soils had lower shoot biomass than controls. Similar results were observed with shoot FW yield (data not shown).

The Fv/Fm values, which represent the maximum potential quantum efficiency of PSII, were less sensitive to treatments than shoot DW yields. There were generally no differences between control plants (growing on the control soil) and lettuces growing on phytomanaged and untreated soils, except for Touro (plants on the UNT soil died before the end of the experiment, so it was not possible to measure Fv/Fm) and Piekary (the UNT plants - but not the PS1 and PS2 plants - had a Fv/Fm value lower than controls) (Fig. 2).

Figure 1: Shoot dry weight yield (g DW plant-1) of lettuces grown on soils from the 10 GRO field trials. The results for ANOVA analysis, with F ratios and p-values, are presented in each plot. Different letters in each barplot indicate significant differences between treatments. No letter indicates lack of significant differences. Treatment abbreviations are presented in Table 1. Note the different scale of the y-axis in the barplots for the UNT, Bettwiesen and Touro samples.

Figure 2: PSII efficiency (expressed as Fv/Fm ratio) of lettuces grown on soils from the 10 GRO field trials. The results for ANOVA analysis, with F ratios and P-values, are presented in each plot. Different lower case letters indicate significant differences between treatments for each site. No letter indicates lack of significant differences. Treatment abbreviations are presented in Table 1.

3.2. Elemental composition of soil pore water (SPW) and lettuce shoots

In general, GRO had no significant effect on soluble TE concentrations (Table 2). The only exceptions were for As at the Piekary site, which decreased (µg L-1) from 38 to 6.4 with a high addition rate of biosolids (PS2), and Cu at the Touro site, which decreased (mg L-1) from 21.4 to 0.04 after compost application. At this site, tobacco (PE) or Salix-Populus (PS) planting after compost amendment produced no additional reduction on SPW Cu concentration. In contrast, at the Bettwiesen and Högbytorp sites, phytoextraction using tobacco or the Salix clone TORA slightly increased SPW Cd and Cr concentrations, respectively. Soluble TE concentrations in soils from the ten contaminated sites were usually higher than in the control soil, with the exception of As in the soils from Arnoldstein and Högbytorp (Table 2).

To better assess the environmental TE availability at each field site, Spearman correlations between soluble and (NH4NO3)-extractable concentrations were performed for the following elements: Cd, Cr, Cu, Ni, Pb and Zn. Significant correlations were found for Cu (rs = 0.72) and Zn (rs = 0.68), two of the most relevant pollutants across this field trial series (supplementary material, Table S6).

Shoot TE concentrations (mg kg-1 DW) varied between treatments for three sites (Table 3): (1) at Touro, the compost amendment strongly reduced shoot Cu concentration (from 278 to less than 10), in line with decreased SPW Cu concentrations, with no additional effect of either phytostabilisation or phytoextraction (a similar situation occurred for shoot Cd, Pb, and Zn concentrations at this site) (2) at Freiberg-Halsbrücke, where lettuces growing on phytomanaged soils displayed higher shoot Pb concentrations than those grown on untreated soils (3.63 vs. 1.87); and (3) at Piekary, where only the highest addition rate of biosolids (PS2) lowered shoot Cd concentrations (2.30 vs. 7.78) as compared to the untreated soil. Irrespective of the treatment applied, mean shoot Cd and Pb concentrations for the Arnoldstein, Freiberg and Piekary lettuces exceeded the EU maximum permitted concentrations (MPC) for edible leafy vegetables, while the MPC for Cd was exceeded also in the Lommel lettuces (Table 3). Regarding Zn, lettuces cultured in the Arnoldstein and Lommel soils had shoot Zn concentrations above the Chinese standards (Zheng et al, 2007). Lettuces grown on the untreated Touro soil (but not on the phytomanaged soils) exceeded MPC values for Cd, Cu, Pb and Zn (Table 3). Nickel soluble concentrations were usually below 20 μg L-1, whereas shoot Ni concentrations were below detection limit for all the samples except those from the Touro UNT soil (supplementary material, Tables S3 and S5).

Concerning shoot TE removal (which integrated potential interaction between shoot DW yield and TE concentrations), phytomanagement treatments (compared with untreated soils) had significant effects at two sites (Table 4). In Arnoldstein, Pb removal was doubled in the PS treatment due to slight (albeit non-significant) increases in shoot biomass and shoot Pb concentrations in phytomanaged soils. In Piekary, there were variations in Cd (increased only in PS1 treatment) and Zn removals (both phytostabilisation treatments produced higher Zn removal than no treatment): these changes were mainly due to a significant effect of phytomanagement on shoot biomass (Fig. 1).

Table 2: Effect of GRO on the TE concentrations in soil pore water. ANOVA analyses have been applied to the relevant contaminants for each site (grey-shaded cells, and see Table 1). Data presented are average values for each site and treatment. Different superscript letters next to values indicate significant differences between treatments for the site (i.e. P-value <0.05). No superscript means no differences between treatments. n.d.: not determined, <d.l.: below detection limit.

| Site | Treatment | Soluble TE concentration (μg L-1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | As | Cd | Cr | Cu | Pb | Zn |
| Arnoldstein | UNT | 0.55 | 32.47 | 0.11 | 10.02 | 18.64 | 3613 |
|  | PS | 0.91 | 14.84 | 0.29 | 12.91 | 9.07 | 1493 |
| Bettwiesen | UNT | 2.72 | 0.15B | 2.16 | 11.50 | <d.l. | 104 |
|  | PE | 4.57 | 0.22A | 4.66 | 17.33 | <d.l. | 144 |
| Freiberg-Halsbrücke | UNT | 15.30 | 4.66 | 0.55 | 11.90 | 2.62 | 216 |
|  | PE | 14.83 | 2.75 | 0.50 | 10.97 | 1.78 | 63 |
| Högbytorp | UNT | 0.96 | <d.l. | 0.23B | <d.l. | <d.l. | <d.l. |
|  | PE | 1.74 | 0.06 | 3.21A | 26.43 | 0.28 | 4.6 |
| INERIS Phytoagglo | PE | 5.42 | <d.l. | 1.36 | 24.53 | 0.59 | 11.4 |
| INERIS Phytosed | PS | 1.59 | 0.29 | 0.27 | 8.61 | <d.l. | 111 |
| Lommel | UNT | 30.60 | 22.17 | 1.14 | 26.73 | 16.12 | 3016 |
|  | PE | 31.77 | 17.20 | 1.25 | 18.63 | 13.43 | 2146 |
| Piekary | UNT | 37.97A | 1.64 | 0.33 | 3.26 | 8.02 | 42.0 |
|  | PS1 | 17.57A | 4.73 | 0.44 | 11.33 | 12.21 | 202 |
|  | PS2 | 6.37B | 0.32 | 0.30 | 3.60 | 2.49 | 63.7 |
| Biogeco | UNT | n.d. | n.d. | n.d. | 193.00 | n.d. | 3.9 |
|  | PE2 | n.d. | n.d. | n.d. | 488.00 | n.d. | 8.0 |
|  | PE1 | n.d. | n.d. | n.d. | 344.23 | n.d. | 40.6 |
|  | PS | n.d. | n.d. | n.d. | 275.00 | n.d. | <d.l. |
| Touro | UNT | n.d. | n.d. | n.d. | 21400.00A | n.d. | 40800 |
|  | UNT+OM | n.d. | n.d. | n.d. | 38.50B | n.d. | 728 |
|  | PE | n.d. | n.d. | n.d. | 158.93B | n.d. | 134 |
|  | PS | n.d. | n.d. | n.d. | 43.27B | n.d. | 26.7 |
| Control soil | CON | 4.70 | <d.l. | 0.37 | 24.00 | <d.l. | 13.1 |

Table 3: Effect of GRO on shoot TE concentrations of lettuces. ANOVA analyses have been applied to the relevant contaminants for each site only (grey-shaded cells, and see Table 1). Data presented are average values for each site and treatment. Different superscript letters next to values indicate significant differences between treatments for the site (i.e. P-value <0.05). No superscript indicates lack of differences between treatments.

| Sites | Treatments |  | Shoot TE concentrations (mg kg-1 DW) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cd | Cu | Pb | Zn |
| Arnoldstein | UNT |  | 16.82 | 9.07 | 3.95 | 295.6 |
|  | PS |  | 17.66 | 10.69 | 5.53 | 210.4 |
| Bettwiesen | UNT |  | 0.47 | 3.92 | 0.74 | 50.6 |
|  | PE |  | 0.53 | 4.68 | 0.85 | 60.4 |
| Freiberg-Halsbrücke | UNT |  | 12.94 | 6.72 | 1.87B | 71.6 |
|  | PE |  | 13.62 | 6.22 | 3.63A | 49.0 |
| Högbytorp | UNT |  | 0.47 | 5.50 | 0.94 | 18.5 |
|  | PE |  | 0.50 | 5.39 | 0.76 | 18.3 |
| INERIS Phytoagglo | PE |  | 0.22 | 8.52 | 1.58 | 27.2 |
| INERIS Phytosed | PS |  | 0.57 | 4.46 | 0.95 | 62.3 |
| Lommel | UNT |  | 19.76 | 6.15 | 0.75 | 523.0 |
|  | PE |  | 18.05 | 5.20 | 1.43 | 312.4 |
| Piekary | UNT |  | 7.78A | 13.94 | 13.26 | 92.8 |
|  | PS1 |  | 9.50A | 8.43 | 10.95 | 124.8 |
|  | PS2 |  | 2.30B | 7.88 | 6.35 | 148.9 |
| Biogeco | UNT |  | 0.27 | 11.13 | 0.41 | 18.5 |
|  | PE2 |  | 0.19 | 8.01 | 0.61 | 23.4 |
|  | PE1 |  | 0.08 | 9.85 | 0.60 | 17.4 |
|  | PS |  | 0.08 | 9.13 | 0.64 | 25.7 |
| Touro | UNT |  | 2.65 | 278.82A | 4.28 | 642.8 |
|  | UNT+OM |  | 0.58 | 8.31B | 0.65 | 79.8 |
|  | PE |  | 0.39 | 8.93B | 0.97 | 62.9 |
|  | PS |  | 0.14 | 9.30B | 0.17 | 37.9 |
| Control soil | CON |  | 0.08 | 4.71 | 0.91 | 20.3 |
| Maximum permitted concentrations in leafy vegetables (mg kg-1, DW) |  |  | 2* | 100** | 3* | 200** |
| Upper critical threshold concentrations (mg kg-1, DW)*** |  |  | 5-10 | 15-30 | 10-20 | 100-300 |

* (EC) No 1881/2006; ** Chinese standards, taken from Zheng et al. (2007)

*** White and Brown (2010)

Table 4: Shoot TE removals by lettuces grown on soils from the ten European GRO field trials. Shaded cells mark the relevant contaminants for each site. Data are average values for each site and treatment. Different superscript letters indicate significant differences between treatments for the site (i.e. P value < 0.05).

| Sites | Treatments | Shoot TE removals (μg plant-1) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cd | Cu | Pb | Zn |
| Arnoldstein | UNT | 26.44 | 13.74 | 5.92B | 441.4 |
|  | PS | 44.27 | 26.63 | 13.87A | 527.1 |
| Bettwiesen | UNT | 2.97 | 24.74 | 4.74 | 320.6 |
|  | PE | 3.38 | 30.00 | 5.21 | 394.8 |
| Freiberg-Halsbrücke | UNT | 42.83 | 23.10 | 6.23 | 243.3 |
|  | PE | 42.92 | 19.75 | 11.84 | 154.3 |
| Högbytorp | UNT | 1.65 | 19.05 | 3.33 | 65.0 |
|  | PE | 1.83 | 19.95 | 2.79 | 66.8 |
| INERIS Phytoagglo | PE | 0.70 | 26.26 | 4.87 | 83.4 |
| INERIS Phytosed | PS | 1.90 | 15.43 | 3.23 | 211.8 |
| Lommel | UNT | 53.45 | 15.32 | 1.82 | 1255.5 |
|  | PE | 63.86 | 18.48 | 4.82 | 1092.0 |
| Piekary | UNT | 8.66B | 15.20 | 14.07 | 103.0C |
|  | PS1 | 20.03A | 17.94 | 22.91 | 261.2B |
|  | PS2 | 6.30B | 21.45 | 17.40 | 395.3A |
| Biogeco | UNT | 0.56 | 24.65 | 0.87 | 41.8 |
|  | PE1 | 0.32 | 38.17 | 2.26 | 66.8 |
|  | PE2 | 0.84 | 37.23 | 2.68 | 109.5 |
|  | PS | 0.24 | 26.22 | 1.52 | 73.9 |
| Touro | UNT | 0.79 | 87.38 | 1.84 | 177.1 |
|  | UNT+OM | 2.03 | 30.05 | 2.23 | 284.4 |
|  | PE | 2.18 | 47.76 | 5.41 | 345.4 |
|  | PS | 0.64 | 42.82 | 0.77 | 174.2 |
| Control soil | CON | 0.39 | 22.33 | 4.21 | 96.3 |

Co-inertia Analyses using SPW pH and soluble and shoot concentrations of trace elements (supplementary material, tables S3 to S5) were carried out on multi-element and on Cu-contaminated sites, respectively. For multi-element contaminated sites, a significant but low relationship was found between SPW and shoot ionome data (RV = 0.28, Fig 3b). For the SPW, the As, Cd, Pb, and Zn concentrations had negative weights on the horizontal axis, whereas on the vertical axis Cu and P concentrations had positive weights, and Mo concentration and pH values had negative weights (Fig 3c, left). Cadmium, Mn and Zn plotted on the same axis and opposite to pH and Mo. This was logical considering the solubility of these four elements is pH dependent, Mo being present as oxyanions, and Cd, Mn and Zn in cationic form. Lead, Cu and Fe also showed a strongly opposing relationship to pH. Phosphorus and As plotted in opposing positions, which may reflect the competitive sorption of phosphates and arsenates in the soils.

Some shoot ionome variables showed similar behaviour to the SPW variables: e.g. Cd and Zn with negative weights on the horizontal axis, whereas P had positive weighting on the vertical axis. Other influencing shoot variables were Cu, Fe and Pb concentrations, all with negative weights on the vertical axis (Fig 3c, right). Interestingly, some trace elements show opposing weightings to macronutrients (Cd, Mn and Zn vs K; Cu, Fe and Pb vs P; Fig 3c, right). This fact may indicate a negative effect of some metal excess (with labile pools in the SPW highly dependent on soil pH) on macronutrient uptake and root-to-shoot transfer in lettuce plants.

There are important differences between sites as the individual samples (i.e. a potted soil and its corresponding lettuce sample) from each site project in different graph sectors (Fig 3d), with two major trends. First, individuals from GRO treatments (i.e. blue and green arrows) are plotted nearer (i.e. more similar) to control individuals (black arrows) than individuals from untreated soils (i.e. red arrows), which may indicate a site-amelioration effect of GRO treatments. Second, shoot ionome values (arrow tips) from different treatments for each site are generally plotted in closer positions (i.e. more similar) than arrow tails (i.e. SPW data). This is especially clear for the Arnoldstein, Bettwiesen and Högbytorp sites (Fig 3d), although there are two exceptions: (1) at Lommel, the arrow tip for one of the three untreated samples (red arrows) is displaced to negative values on the coinertia horizontal axis (i.e. higher shoot Cd and Zn concentrations) and (2) at Piekary, the arrow tips for two untreated samples are displaced to negative values on the coinertia vertical axis (i.e. lower shoot P concentration and higher shoot Cu, Fe and Pb concentrations) (Figs 3c and 3d).

Figure 3: Overview of CoInertia Analysis (CoIA) performed on the SPW and lettuce shoot data for multi-contaminated sites. (A) Correlation circles showing the projections of the PCA axes from SPW data (left) and shoot data (right) onto the axes of the coinertia analysis. (B) Histogram of Permutation test showing the random distribution of RV values and the actual RV coefficient (grey diamond). (C) Representation of the combinations of SPW (left) or shoot (right) variables to defined coinertia axes. (D) CoIA results (separated for each site, all graphs derived from the same analysis). Each arrow corresponds to one individual, i.e. a potted soil and its corresponding lettuce sample. The arrow tail is individual position of the sample described by SPW variables; the arrow tip corresponds to the individual position described by shoot variables. Arrow length informs about the differences between SPW and shoot ionome values for each individual: the shorter the arrow, the stronger the relationship between both datasets. Treatments are indicated by arrow colours: red-untreated, green-phytoextraction, blue-phytostabilisation. Different blue colours in Piekary indicate different phytostabilisation treatments (light blue: PS1; dark blue: PS2). The position of uncontaminated samples in the CoIA is indicated by black arrows in each graph. Colours refer to the online version only.

Figure 4: Overview of CoInertia Analysis (CoIA) performed on the SPW and lettuce shoot data from Cu-contaminated sites. (A) Correlation circles showing the projections of the PCA axes from SPW data (top) and shoot data (bottom) onto the axes of the coinertia analysis. (B) Histogram of Permutation test showing the random distribution of RV values and the actual RV coefficient (grey diamond). (C) Representation of the combinations of SPW (left) or shoot (right) variables to defined coinertia axes. (D) CoIA results (separated for each site, all graphs derived from the same analysis). Each arrow corresponds to one individual, i.e. a potted soil and its corresponding lettuce sample. The arrow tail is individual position of the sample described by SPW variables; the arrow tip corresponds to the individual position described by shoot variables. Arrow length informs about the differences between SPW and shoot ionome values for each individual: the shorter the arrow, the stronger the relationship between both datasets. Treatments are indicated by arrow colours: red-untreated, orange- amended and unplanted, green-phytoextraction, blue-phytostabilisation. Different green colours in Biogeco indicate different phytoextraction treatments (light green: PE1; dark green: PE2). The position of uncontaminated samples in the CoIA is indicated by black arrows in each graph. Colours refer to the online version only.

For Cu-polluted sites (Fig 4), there was a clear relationship between the SPW and shoot ionome data (RV = 0.77, Fig 4b). Moreover, SPW and shoot ionome variables had similar weights on the CoIA axes: TE concentrations (and pH for the SPW) plot largely on the horizontal axes, whereas K and P, being nutrients for plant development, plot on the vertical axis (Fig 4c). Accordingly, the Touro UNT samples, with extreme TE (mainly Cu) concentrations both in SPW and in shoots, show strongly negative values on the horizontal axis, plotting a significant distance away from the other samples, which show positive values on the horizontal axis (Fig 4d). There is a similar pattern in both Cu-contaminated sites: samples from GRO treatments (blue, green and yellow arrows) are plotted closer to control samples (black arrows) than UNT samples (red arrows). However, the GRO effect differs at each site: at Touro, the transition occurs along the horizontal axis (i.e. reduction in concentrations of TE), whereas at Biogeco it occurs along the vertical axis (i.e. increase in nutrient concentrations in the SPW and shoots) (Fig 4d). Moreover, for Touro, the phytostabilisation and phytoextraction individuals occupy almost the same position (well away from the UNT individuals) in the co-inertia space (i.e. they have similar values both in SPW and shoot ionomes). For Biogeco, the phytostabilisation individuals are placed next to the UNT ones and the phytoextraction treatments with two compost dressings occupy a similar position as individuals for the control soil (i.e. phytostabilisation samples are nutrient-depleted, whereas compost application and fertilisation of the crop rotation improved progressively the nutrient status in phytoextraction samples) (Fig 4d).

4. Discussion

The potential of GRO to ameliorate TE-contaminated soils is being investigated in a European network of phytomanaged field trials (i.e. Greenland project, Kidd et al. 2015). Three years after the first inter-laboratory risk assessment testing (reported in Kumpiene et al. 2014), soils were collected again for a second testing. We have used TE concentrations in the SPW and lettuce shoots, as well as lettuce shoot DW yield and PSII activity, as proxies for the improvement of some soil functions (e.g. provisioning, regulation of metal(loid) exposure).

If we consider the changes in soluble and shoot TE concentrations, the GRO effects are quite modest, with a significant effect only in the most contaminated soils (i.e. Touro and Piekary). Although the use of SPW TE concentrations may not reflect the environmental TE bioavailability in soils (Marchand et al. 2016), SPW sampling and analysis was a relevant method to predict this parameter in multi-element contaminated soils (Qasim et al. 2016). Kumpiene et al. (2014), using soils from some of the sites analysed in this study, showed that H2O-extracted TE concentrations had more similar patterns of variation between treatments than other extractable TE concentrations (e.g. NH4NO3 or EDTA) despite the H2O extractable TE concentrations being lower than the chemical-based extractions. Correlation analyses of our SPW data with NH4NO3 datasets from the field trials revealed significant correlations for two relevant contaminants (Cu and Zn, Table S6).

Multivariate Co-Inertia Analysis has shown a significant correlation between SPW and shoot ionomes, although for each site shoot ionomes are less variable (i.e. they are less dispersed in the co-inertia space) than soluble ionomes. Whereas soil factors affect labile TE pools, plant physiological processes (mainly related to the transcriptional control of membrane transporters) regulate metal(loid) uptake into the symplast and subsequent translocation to plant shoots (Pilon et al. 2009, Mench et al. 2010). These mechanisms allow plants to keep stable ionomes in spite of varying soil conditions, and in some plant species tolerance strategies have evolved to keep ion homeostasis even under excessive TE concentrations in soils (Baker 1981, Małachowska-Jutsz & Gnida 2015). To our knowledge, lettuce has not been described as a metal tolerant species, but it can accumulate elevated Cd concentrations on shoots without toxicity symptons such as chlorosis or necrosis (Zorrig et al. 2013) and between-cultivar differences in Cd and Zn accumulation have been found (Crews & Davies 1985, Costa & Morel 1994, Fontes et al. 2014). The lower variability of shoot ionomes (Table 3) indicates that lettuce has efficient mechanisms of ion homeostasis to deal with some of the contaminated soils used. Previous research reported a certain tolerance to salinity in lettuce (Zapata et al. 2003, Nasri et al. 2011) which may indicate a capacity for response to trace elements. Conversely, in those phytotoxic contaminated soils where exposure to some metal(loid)s surpasses metabolic limits, the response of shoot ionomes is erratic. This was the case for two out of three untreated samples from the Lommel site and from the Piekary sites, or the more extreme case of the untreated Touro soils.

Multivariate CoIA plots (considering all the elements of SPW and shoot ionomes) showed a gradient of soil amelioration from untreated to GRO-managed soils (i.e. in the co-inertia space the GRO samples are placed nearer to the control samples than the untreated samples). This gradient is especially pronounced in the brownfield and mine tailing sites (i.e. Biogeco, Piekary and Touro).

Soil TE contamination at a certain site usually involves several chemical elements, usually in interaction with other soil factors such as nutrients, pH, organic matter, clays, etc., making interpretation of soil contamination and its potential impacts complex. Here, the application of multivariate techniques (routinely used in plant sciences and community ecology, e.g. Horáčková et al. 2016) to standard SPW and Plantox test data has allowed much clearer assessment of TE processes and impacts, and discrimination of GRO effectiveness, in these soil/plant systems. Our findings indicate that application of similar methods may improve assessment of soil/plant processes and GRO (and wider remediation / site management technology) risk management effectiveness at other contaminated sites.

PSII activity (measured as Fv/Fm ratio) has been indicated as a relevant endpoint for plant stress (Maxwell & Johnson, 2000). However, differences in this plant parameter between soil treatments were only observed in one out of the ten sites. This may be because Fv/Fm measures the state of the PSII only, and changes in Fv/Fm may not be detected if the stress occurs in other plant parts such as the roots (Murchie & Lawson 2013). Based on shoot ionome, lettuce plants have a certain capacity to regulate TE uptake and concentrations in shoots so the stress effects on PSII may have been reduced. Interestingly, the Piekary UNT samples, where soil conditions have surpassed plant limits (i.e. two out of three samples had diverging shoot ionomes), showed a Fv/Fm value lower than phytomanaged and control samples. However, for other sites where upper critical threshold concentrations in shoots were exceeded (i.e. Cd for the Arnoldstein, Freiberg, and Lommel lettuces, Zn for the Arnoldstein, Lommel and –untreated- Touro lettuces, and Cu for the –untreated- Touro lettuces (Table 3)) no response in Fv/Fm was observed (except in the dead untreated Touro plants). Thus, the Fv/Fm endpoint showed in our study a lack of sensitivity to detect physiological effects of excessive metal(loid) exposure at the TE-contaminated sites. Similarly, Marchand et al. (2016) did not find any differences in Fv/Fm in Ranunculus acris L. cultivated in a gradient of soil pollution. Moreover, Vaillant et al. (2005) found a species-dependent reduction in Fv/Fm in four Datura species grown under hydroponics with increasing Zn concentrations. These findings suggest that the usefulness of Fv/Fm photosyntethic parameter on ecotoxicological studies is highly dependent on the model plant species chosen; i.e. the use of more tolerant species or cultivars would reduce the sensitivity of this parameter to soil toxicity.

In the case of lettuce, Zorrig et al. (2013) found no symptoms of chlorosis or necrosis in plants exposed to Cd treatments in hydroponics, even in the case of biomass reduction. Thus, Fv/Fm may not be useful in studies with lettuce. Moreover, and considering the differences in Cd or Zn accumulation between lettuce cultivars (Crews & Davies 1985, Costa & Morel 1994, Fontes et al. 2014), we suggest a screening of lettuce cultivars should be done to select a sensitive one to be used as a standard in soil toxicity assessments. Its use would improve the usefulness of lettuce as bioindicator and a proper comparison of results obtained on different studies. However availability of commercial cultivars is changing to maintain resistance to pathogens.

GRO implementation had a positive impact on shoot DW yield (Fig. 1). In four out of the ten sites, the plants growing on phytomanaged soils showed higher biomasses than those on untreated soils, and at four sites the GRO yielded a similar (or higher – Bettwiesen –) shoot biomass as the controls. The PSII performances indicate that these changes in shoot biomass are not caused by changes in the photosynthesis capacities of the plants. Instead, soil amelioration driven by GRO implies that plants need to allocate less resources to mechanisms of TE detoxification (such as active efflux into apoplasts, vacuolar compartimentalization, chelation in cytosol by various ligands or repair and protection of plasma membranes, among others; Yadav 2010, Singh et al. 2015, Luo et al. 2016, Sharma et al. 2016), leaving more resources to be dedicated to growth.

We found a relationship between soluble and shoot TE concentrations (assessed with CoIA) and shoot biomass: the closer individual samples (i.e. a potted soil and its corresponding lettuce sample) are to the controls in the co-inertia space, the higher the shoot biomass (Fig. 3 and 4). The most extreme examples are given by the samples from Bettwiesen (UNT and PE), Biogeco (PE2) and Touro (PE and PS), which overlap control samples in the co-inertia space and have similar shoot biomass as the controls (or higher biomass in the case of Bettwiesen –UNT and PE- and Touro PE).

The ten analysed sites are either TE-contaminated agricultural soils (five sites) or brownfields/mine tailings (five sites). The UNT soils from three out of the latter five sites are usually the worst performing in term of lettuce growth and plant performances at field trials (Table S7). However, in these brownfield/mine sites, GRO had a more ameliorating effect and in two cases (Biogeco and Touro) similar shoot biomasses were found in phytomanaged soils as controls.

For the Biogeco and Touro Cu-contaminated soils, both phytostabilisation and phytoextraction options were implemented thus allowing their comparison. The best results (assessed as shoot DW yield) were achieved by compost amendment followed by phytoextraction. The addition of compost added both organic matter and nutrients to the soil, and at Touro it also reduced metal availability (i.e. Cu, Table 2), thus improving soil quality and favouring lettuce growth.

Phytoextraction on TE-contaminated soils involves the cultivation of plants accumulating metal(loid)s in the harvested plant parts, usually leaves or shoots. Each crop also implies that nutrients contained in harvested plant parts are also removed from the phytomanaged plot. Depending on initial status of soils at contaminated sites, repeated harvest for phytoextraction (or for phytostabilisation combined with SRC) may reduce the soil fertility (i.e. crop production) in the mid-long term. This effect would be clearer for contaminated soils with a poor nutrient stock. Our study included two contrasting sites where phytoextraction with tobacco or tobacco/sunflower rotation was applied: Bettwiesen (nutrient rich, high CEC) and Biogeco (nutrient poor, low CEC). Considering Biogeco, lettuce shoot DW yield and K and P concentrations in shoots and SPW were similar for the PE1 (one compost dressing in 2008) and the UNT soils, and lower than the PE2 soil (two compost dressings in 2008 and 2013). In contrast at Bettwiesen, these parameters were similar for the UNT and PE soils, but higher than Control soil. Thus, depending on the initial soil fertility, the implementation of long-term phytoextraction (and also phytostabilisation with SRC) options should include recurrent fertilisation in the maintenance procedure to ensure GRO performance (i.e. to maintain crop production with common shoot nutrient concentrations).

Acknowledgements: We acknowledge financial support by the European Commission within the 7th Framework program (FP7-KBBE-266124). C. Quintela-Sabaris was financed by a contract of the Euskampus Fundazioa (www.euskampus.eu) and a Postdoctoral contract financed by the French National Research Agency through the national program “Investissements d’avenir” with the reference ANR-10-LABX-21-01 / LABEX RESSOURCES21. P. Kidd would like to thank the financial support of the Spanish Ministerio de Economía y Competitividad (CTM2012-39904-C02-01) and FEDER. W. Friesl-Hanl would like to thank the Austrian Federal Ministry of Agriculture, Forestry, Environment and Water Management for funding the projects INTERLAND (grant number A220001) and NUTZRAUM (grant number A62000) which partly financed a long-term study in Austria as a basis for improving field experiences. V. Bert would like to thank ANR, VNF and ADEME for financing the BIOFILTREE and PHYTOSED Ec1 project. The Biogeco site was additionally supported by ADEME (Angers, France), the Aquitaine Region Council, (Bordeaux, France), the COST Actions 859 and FA905 (http://w3.gre.ac.uk/cost859/; http://www.umb.no/costaction/), the Euskampus fundazioa (www.euskampus.eu), and the ANR project Phytochem (http://www.agence-nationale-recherche.fr/projetanr/?tx lwmsuivibilan pi2%5BCODE%5D=ANR-13-CDII-0005). The UMR Biogeco is a member of the INRA Ecotoxicologist network, ECOTOX. N. Oustriere is grateful to Ademe for supporting her PhD grant.

References

Aronsson P, Dahlin T, Dimitriou I (2010) Treatment of landfill leachate by irrigation of willow coppice–plant response and treatment efficiency. Environ Pollut 158:795-804.

Baker AJ (1981) Accumulators and excluders-strategies in the response of plants to heavy metals. J Plant Nutr 3:643-654.

Bert V, Cadière F (2015) Phytotechnologies is soil bioengineering, Proceeding AGéBio international conference, soil and water bioengineering: skills, regulations and benefits, November 23-25, Lyon, France. Available at http://www.agebio.org/IMG/pdf/Valerie_Bert_Ineris.pdf

Bes C (2008) Phytoremédiation des sols d’un site de traitement du bois contaminés par le cuivre. PhD thesis, University of Bordeaux 1, Talence. 293 p. http://ori-oai.u-bordeaux1.fr/pdf/2008/BES_CLEMENCE_2008.pdf

Bolhàr-Nordenkampf HR, Long SP, Baker NR, Oequist G, Schreiber U, Lechner EG (1989) Chlorophyll fluorescence as a probe of the photosynthetic competence of leaves in the field: A review of current instrumentation. Funct Ecol 3:497-514.

Butler WL, Kitajima M (1975) Fluorescence quenching in photosystem II of chloroplasts. Biochim Biophys Acta 376:116-125.

Chessel D, Dufour AB, Thioulouse J (2004) The ade4 package-I- One-table methods. R News 4:5-10.

Cundy AB, Bardos RP, Church A, Puschenreiter M, Friesl-Hanl W, Müller I, Neu S, Mench M, Witters N, Vangronsveld J (2013). Developing principles of sustainability and stakeholder engagement for “gentle” remediation approaches: The European context. J Environ Manage 129:283-291.

Cundy AB, Bardos RP, Puschenreiter M, Mench M, Bert V, Friesl-Hanl W, Müller I, Li XN, Weyens N, Witters N, Vangronsveld J (2016) Brownfields to green fields: Realising wider benefits from practical contaminant phytomanagement strategies. J Environ Manage 184:67-77.

Defra, 2012. Environmental Protection Act 1990: Part 2A, Contaminated Land Statutory Guidance April 2012. PB13735 Defra, London, UK. http://www.defra.gov.uk/publications/2012/04/10/pb13735contaminated-land/ (accessed Jan 2017).

Dietzsch A (2011) Nutzung kontaminierter Böden. Schriftenreihe des Sächsischen Landesamts für Umwelt. Landwirtschaft und Geologie. Dresden: Heft 19/2011; [https://publikationen.sachsen.de/bdb/artikel/14994/documents/17993]

DIN ISO 19730 (2008). Extraktion von Spurenelementen mit Ammoniumnitratlösung. Berlin: Deutsches Institut für Normung Hrsg.

Dray S, Dufour AB, Chessel D (2007) The ade4 package - II: Two-table and K-table methods. R News 7:47-52.

EEA (European Environment Agency) (2014) Progress in management of contaminated sites, CSI 015, LSI 003 Copenhagen, Denmark. Available at http://www.eea.europa.eu/data-and-maps/indicators/progress-in-management-of-contaminated-sites-3/assessment/ accessed April 26, 2016

FAO and ITPS. 2015. Status of the World’s Soil Resources (SWSR) – Main Report. Food and Agriculture Organization of the United Nations and Intergovernmental Technical Panel on Soils, Rome, Italy. ISBN 978-92-5-109004-6.

Fontes RLF, Pereira JMN, Neves JCL (2014) Uptake and translocation of Cd and Zn in two lettuce cultivars. An Acad Bras Cienc 86:907-922. doi: 10.1590/0001-37652014117912

Friesl W, Friedl J, Platzer K, Horak O, Gerzabek MH (2006) Remediation of contaminated soils in the vicinity of a former Pb/Zn smelter in Austria: batch, pot, and field experiments. Environ Pollut 144:40–50.

Friesl-Hanl W, Platzer K, Horak O, Gerzabek MH (2009) Immobilising of Cd, Pb, and Zn contaminated arable soils close to a former Pb/Zn smelter: a field study in Austria over 5 years. Environ Geochem Health 31:581–94.

Herzig R, Nehnevajova E, Pfistner C, Schwitzguebel, J. P, Ricci A, Keller C. 2014. Feasibility of labile Zn phytoextraction using enhanced tobacco and sunflower: Results of five- and one-year field-scale experiments in Switzerland. Int J Phytoremediation 16:735–754.

Horáčková M, Řehounková K, Prach K (2016). Are seed and dispersal characteristics of plants capable of predicting colonization of post-mining sites? Environ Sci Pollut Res Int, 23(14):13617-13625. doi: 10.1007/s11356-015-5415-5

Hua Y, Heal KV, Friesl-Hanl W (2017) The use of red mud as an immobiliser for metal/metalloid-contaminated soil: A review. J Haz Mat 325:17-30.

Janssen J (2015) Metal phytoextraction: the potential of high biomass crops and improving efficiency using short rotation coppice (SRC) of willow. PhD thesis, Univ. Hasselt, 295 p.

Kidd PS, Prieto-Fernández A, Rodríguez-Garrido B, Álvarez-López V, Trasar-Cepeda C, Touceda-González M, Mench M, Puschenreiter M, Crespo-Queiruga C, Macías-García F (2012) Establishing phytostabilization field plots in Cu rich mine tailings: an initial evaluation of plant species and soil amendments. Proceedings of the 4th International Congress Eurosoil 2012. Soil Science for the benefit of mankind and environment.

Kidd PS, Mench M, Álvarez-López V, Bert V, Dimitriou I, Friesl-Hanl W, Herzig R, Janssen JO, Kolbas A, Müller I, Neu S, Renella G, Ruttens A, Vangronsveld J, Puschenreiter M (2015) Agronomic practices for improving gentle remediation of trace element-contaminated soils. Int J Phytoremediation 17:1005-1037.

Kolbas A, Mench M, Herzig R, Nehnevajova E, Bes CM (2011) Copper phytoextraction in tandem with oilseed production using commercial cultivars and mutant lines of sunflower. Int J Phytoremediation 13:55–76.

Kolbas A (2012) Phenotypic traits and development of plants exposed to trace elements; use for phytoremediation and biomonitoring. PhD thesis, Sciences & Environnements, spécialité Ecologie Evolutive, Fonctionnelle et des Communautés, Université Bordeaux 1, Talence, France.

Kolbas A, Marchand L, Herzig R, Nehnevajova E, Mench M (2014). Phenotypic seedling responses of a metal-tolerant mutant line of sunflower growing on a Cu-contaminated soil series: potential uses for biomonitoring of Cu exposure and phytoremediation. Plant Soil 376:377-397.

Kumpiene J, Bert V, Dimitriou I, Eriksson J, Friesl-Hanl W, Galazka F, Herzig R, Janssen JO, Kidd P, Mench M, Müller l, Neu S, Oustriere N, Puschenreiter M, Renella G, Roumier PH, Siebielec G, Vangronsveld J, Manier N (2014) Selecting chemical and ecotoxicological test batteries for risk assessment of trace element-contaminated soils (phyto)managed by Gentle Remediation Options (GRO). Sci Tot Environ 496:510-522. doi: 10.1016/j.scitotenv.2014.06.130

Legendre P, Legendre L (2012) Numerical Ecology 3rd Edition. Developments in Environmental Modelling Vol. 24. Elsevier, Amsterdam, The Netherlands, 969 pp.

Luo ZB, He J, Polle A, Rennenberg H (2016) Heavy metal accumulation and signal transduction in herbaceous and woody plants: Paving the way for enhancing phytoremediation efficiency. Biotechnology Advances 34:1131–1148. doi: 10.1016/j.biotechadv.2016.07.003

Małachowska-Jutsz A, Gnida A (2015) Mechanisms of stress avoidance and tolerance by plants used in phytoremediation of heavy metals. Arch Environ Prot 41: 104-114

Marchand L, Lamy P, Bert V, Quintela-Sabaris C, Mench M (2016) Potential of Ranunculus acris L. for biomonitoring trace element contamination of riverbank soils: photosystem II activity and phenotypic responses for two soil series. Environ Sci Pollut Res 23:3104-3119

Maxwell K, Johnson GL (2000) Chlorophyll fluorescence –a practical guide. J Exp Bot 51:659-669

Mench M, Lepp N, Bert V, Schwitzguébel JP, Gawronski SW, Schröder P, Vangronsveld J (2010) Successes and limitations of phytotechnologies at field scale: outcomes, assessment and outlook from COST Action 859. J Soils Sediments 10:1039-1070.

Meuser H (2013) Soil Remediation and Rehabilitation: Treatment of Contaminated and Disturbed Land. Environmental Pollution 23, Springer Science+Business Media Dordrecht. doi: 10.1007/978-94-007-5751-6_1

Murchie EH, Lawson T (2013) Chlorophyll fluorescence analysis: a guide to good practice and understanding some new applications. J Exp Bot 64:3983-3998.

Nasri N, Kaddour R, Rabhi M, Plassard C, Lachaal M (2011) Effect of salinity on germination, phytase activity and phytate content in lettuce seedling. Acta Physiol Plant 33:935-942.

Panagos P, Van Liedekerke M, Yigini Y, Montanarella L (2013) Contaminated sites in Europe: review of the current situation based on data collected through a European network. J Environ Public Health, 2013.

Pilon M, Cohu CM, Ravet K, Abdel-Ghany SE, Gaymard F (2009) Essential transition metal homeostasis in plants. Curr Opin Plant Biol 12:347-357.

Puschenreiter M, Mench M, Bert V, Kumpiene J, Kidd P, Cundy A, Müller I, Vangronsveld J, Renella G, Friesl-Hanl W, Siebielec G, Herzig R, Müller I, Dimitriou J, Quiroga Troncosa X, Bajorek R, Lemaitre P, Serani Loppinet A (2014) Best practice guidance for practical application of gentle remediation options (GRO). FP7 EU Project GREENLAND – Gentle remediation of trace element contaminated land (FP7-KBBE-266124, Greenland) (www.greenland-project.eu/).

Qasim B, Motelica-Heino M, Joussein E, Soubrand M, Gauthier A (2016) Diffusive gradients in thin films, Rhizon soil moisture samplers, and indicator plants to predict the bioavailabilities of potentially toxic elements in contaminated technosols. Environ Sci Pollut Res 23:8367–8378. doi: 10.1007/s11356-015-5975-4

Ruttens A, Mench M, Colpaert JV, Boisson J, Carleer R, Vangronsveld J (2006) Phytostabilization of a metal contaminated sandy soil. I: Influence of compost and/or inorganic metal immobilizing soil amendments on phytotoxicity and plant availability of metals. Environ Pollut 144:524-532.

Ruttens A, Boulet J, Weyens N, Smeets K, Adriaensen K, Meers E, Van Slycken S, Tack F, Meiresonne L, Thewys T, Witters N, Carleer R, Dupae J, Vangronsveld J (2011) Short rotation coppice of willows and poplars as energy crops on metal contaminated agricultural soils. Int J Phytoremediation 13:194–207.

Sharma SS, Dietz KJ, Mimura T (2016) Vacuolar compartmentalization as indispensable component of heavy metal detoxification in plants. Plant, Cell and Environment 39:1112–1126. doi 10.1111/pce.12706

Singh S, Parihar P, Singh R, Singh VP, Prasad SM (2015) Heavy metal tolerance in plants: Role of transcriptomics, proteomics, metabolomics, and ionomics. Front Plant Sci 6, Article 1143. doi: 10.3389/fpls.2015.01143

Stuczynski T, Siebielec G, Daniels WL, McCarty G, Chaney RL (2007) Biological aspects of metal waste reclamation with biosolids. J Environ Qual 36:1154-1162.

Tordoff GM, Baker AJM, Willis AJ (2000) Current approaches to the revegetation and reclamation of metalliferous mine wastes. Chemosphere 41:219-228.

Touceda-Gonzalez M, Álvarez-Lopez V, Prieto-Fernandez A, Rodriguez-Garrido B, Trasar-Cepeda C, Mench M, Puschenreiter M, Quintela-Sabaris C, Macias-Garcia F, Kidd PS (2017) Aided phytostabilisation reduces metal toxicity, improves soil fertility and enhances microbial activity in Cu-rich mine tailings. J Environ Manage 186:301-313. doi: 10.1016/j.jenvman.2016.09.019

Vaillant N, Monnet F, Hitmi A, Sallanon H, Coudret A (2005) Comparative study of responses in four Datura species to a zinc stress. Chemosphere 59:1005-1013.

White PJ, Brown PH (2010). Plant nutrition for sustainable development and global health. Ann Bot 105:1073-1080.

Yadav SK (2010) Heavy metals toxicity in plants: an overview on the role of glutathione and phytochelatins in heavy metal stress tolerance of plants. S Afr J Bot 76:167-179.

Yao Z, Li J, Xie H, Yu C (2012) Review on remediation technologies of soil contaminated by heavy metals. Procedia Environ Sci 16:722-729.

Zapata PJ, Pretel MT, Amorós A, Botella MA (2003) Changes in ethylene evolution and polyamine profiles of seedlings of nine cultivars of Lactuca sativa L. in response to salt stress during germination. Plant Sci 164:557-563.

Zheng N, Wang Q, Zheng D (2007) Health risk of Hg, Pb, Cd, Zn and Cu to the inhabitants around Huludao Zinc Plant in China via consumption of vegetables. Sci Tot Environ 383:81-89.

Zorrig W, El Khouni A, Ghnaya T, Davidian JC, Abdelly C, Berthomieu P (2013). Lettuce (Lactuca sativa): a species with a high capacity for cadmium (Cd) accumulation and growth stimulation in the presence of low Cd concentrations. J Hortic Sci Biotechnol 88:783-789.

SUPPLEMENTARY MATERIAL

Table S.1: Sites and treatments.

| Management strategy / Sites | Country | Main contaminants | Description | References |
| --- | --- | --- | --- | --- |
| (Aided) Phytoextraction |  |  |  |  |
| Bettwiesen | Switzerland | Cd, Zn, Pb | Plots are established on an agricultural field on old landfill. Somaclonal variants and mother lines of tobacco are used for biomass production and TE removal from soil. | Herzig et al (2014) |
| Freiberg-Halsbrücke | Germany | As, Cd, Pb | Agricultural polluted soil. Long term field trial; a short rotation coppice (SRC) of Salix clone Tora is used for biomass production and removal of TE from soil | Dietzsch (2011) |
| Högbytorp | Sweden | (Cd), Cr, Zn | Long term field trial on agricultural field next to a landfill; SRC of Salix clone Tora is watered using landfill leachate. Plants are intended for biomass production and removal of TE from soil | Aronsson et al. (2010) |
| Ineris Creil-Phytoagglo | France | Cd, Zn, Pb | Polluted brownfield soil planted with local cultivar of Salix viminalis and hyperaccumulator Arabidopsis halleri. | Bert and Cadière (2015) |
| Lommel | Belgium | Cd, Zn, Pb | Polluted agricultural soil. Long term field trial; SRC of Salix schwerinii × Salix viminalis clone Tora is used for biomass production and removal of TE from soil. | Ruttens et al. (2011) |
| Biogeco | France | Cu | Polluted industrial soil (wood preservation). Long term field trial (since 2008); aided Phytoextraction by amendment with compost (5% w/w) and dolomitic limestone (0.2% w/w) (OMDL = PE1) and crop rotation with tobacco and sunflower. A second compost dressing was applied in 2013 on half of the field trial (OM2DL = PE2) | Kolbas et al. (2011); Kolbas (2012) |
| Touro | Spain | Cu | Copper mine tailings amended with compost and cultivated with somaclonal variants and mother lines of tobacco are used for biomass production and TE removal from soil. | Kidd et al (2015) |
| In situ stabilization/ phytoexclusion |  |  |  |  |
| Arnoldstein | Austria | As, Cd, Zn, Pb | Polluted agricultural soil. Field plots; gravel sludge (GS) and siderite-bearing materials (SI) were combined to immobilize TE in contaminated soil. The site was cultivated with maize and barley cultivars with contrasting phenotype regarding metal uptake | Friesl et al. (2006); Friesl-Hanl et al. (2009) |
| Ineris Fresnes-sur-Escaut Phytosed | France | Cd, Zn, Pb, Cu | Technosol developed on dredged-sediment and amended with basic slags (Optiscor). Grass cover with Deschampsia cespitosa, combined with Short rotation coppice (as energy crops) with Salix cv Tordis ((Salix viminalis x Salix Schwerinii) x Salix viminalis = breeding Tora x Z. Ulv) Salix cv Inger (Salix triandra x Salix viminalis = breeding Jorr x siberian Klon) | Kidd et al (2015) |
| Piekary | Poland | Zn, Cd, Pb | Highly polluted post-industrial site. Long term in situ stabilization/phytoexclusion experiments established in 1995 and 1997; combinations of two amendments, by-product limestone (L) and municipal biosolids (B), at two rates (low — L and high — H) were tested for their efficiency to immobilize TE (Zn, Cd, Pb). The site was vegetated with a mixture of grasses | Stuczynski et al. (2007) |
| Aided phytostabilization |  |  |  |  |
| Biogeco | France | Cu | Polluted industrial site (wood preservation). One plot (3 m × 6 m) amended in 2006 with 2% of zerovalent iron grit and 5% of compost (OMZ) and cultivated with various plants (i.e. Cytisus striatus, Populus nigra, Rumex acetosella, Agrostis capillaris, Agrostis gigantea, etc.) | Bes (2008) |
| Touro | Spain | Cu | Copper mine tailings amended with compost, seeded with Agrostis capillaries and cultivated with different tree species (Populus nigra, Salix viminalis and S. caprea) for biomass production. | Kidd et al (2015); Touceda et al (2017) |

Table S2: Overview of properties of control garden soil and of untreated soils in each field trial. Trace elements refer to total concentrations and are expressed in mg kg-1

| Site/Country | Control soil France | Arnoldstein Austria | Bettwiesen Switzerland | Biogeco France | Freiberg Germany | Högbytorp Sweden | Ineris Fresne sur Escaut Phytosed France | Ineris CAC Phytoagglo France | Lommel Belgium | Piekary Poland | Touro Spain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH | 7.9 | 5.0 | 6.8 | 7.35±0.09 | 4.9 | 6.8 | 7.5 | 8.12 ± 0.04 | 5.8 | 8.1±0.0 | 3.34 ± 0.02 |
| OM (%) | 4.0 | 2.6 | 3.6 | 1.34±0.04 | 2.5 |  | 31.0 | 4 | 32.8 | 7.8±2.7 | 2.49 ± 0.03 |
| CEC (cmol(+)kg-1) | 16.1 | 5.0 | 32.8 | 2.53±0.24 | 8.4 |  | 23.5 | 12.42 | 6.2 | 50.4±2.2 | 9.38 ± 0.91 |
| As | 3.6 | 36.5±2.7 | 11.0±0.8 | 7.39±0.37 | 128±18 | 9.8 | 42.9±3.5 | 13±0.9 | 7.4±0.8 | 1040 | 11.34 ± 0.98 |
| Cd | 0.27 | 6.1±0.3 | 1.2±0.3 | 0.08±0.01 | 3.4±0.4 | 0.30 | 9.39±0.9 | 2.9±3.7 | 10.9±0.9 | 227.1 | 0.29 ± 0.13 |
| Cr | 18 | 30.9±3.9 | 168±38 | 21.1±0.7 | 29.7±2.11 | 45.2 | 99.7±4.2 | 39.7± 4.4 | 4.8±1.0 | 21.2 | 84.1 ± 0.3 |
| Cu | 21 | 58.6±4.8 | 40.7±2.4 | 964±19 | 28.3±4.5 | 56.8 | 110±7.2 | 99.8 ± 24 | 26.7±1.5 | 2059 | 433.0 ± 8.3 |
| Pb | 24 | 791±70 | 59.5±10.2 | 25.9±3.9 | 302±53 | 20.7 | 956±88.6 | 262±19 | 238±20 | 10511.1 | 13.1 ± 1.8 |
| Zn | 51 | 712±58 | 1698±411 | 37±1.6 | 255±38 | 123.7 | 6089±825 | 1117±823 | 682±64 | 25364.3 | 120.2 ± 1.1 |

Table S3: Values of shoot parameters. For each site and treatment the values of FW, DW (expressed in g plant-1), the PSII efficiency (expressed as Fv/Fm) and the concentration of trace elements and nutrients (expressed in mg kg-1) are presented. For each column the maximum values are indicated in bold type. Empty cells indicate values not measured. <d.l.: below detection limit.

| Site | Treatment | Fv/Fm | FW (g) | DW (g) | Ca | K | Mg | P | Cd | Co | Cu | Fe | Mn | Ni | Pb | Zn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arnoldstein | UNT | 0.828 | 11.7 | 1.50 | 8472 | 26772 | 2539 | 1104 | 16.82 | 0.08 | 9.07 | 108.2 | 64.9 | <d.l. | 3.94 | 295.65 |
|  | PS | 0.832 | 20.9 | 2.51 | 8610 | 30630 | 3006 | 1828 | 17.66 | 0.08 | 10.69 | 83.2 | 84.4 | <d.l. | 5.52 | 210.40 |
| Bettwiesen | UNT | 0.801 | 53.0 | 6.37 | 9098 | 42791 | 1743 | 2694 | 0.47 | 0.28 | 3.92 | 57.3 | 12.8 | <d.l. | 0.74 | 50.57 |
|  | PE | 0.823 | 60.6 | 6.54 | 8962 | 47759 | 2038 | 3102 | 0.53 | 0.38 | 4.68 | 81.2 | 17.7 | <d.l. | 0.84 | 60.39 |
| Freiberg-Halsbrücke | UNT | 0.829 | 32.5 | 3.38 | 8336 | 42088 | 2043 | 2148 | 12.94 | 0.16 | 6.72 | 58.3 | 95.6 | <d.l. | 1.87 | 71.60 |
|  | PE | 0.821 | 26.6 | 3.23 | 9167 | 44153 | 2471 | 2311 | 13.62 | 0.14 | 6.22 | 82.4 | 153.3 | <d.l. | 3.63 | 49.03 |
| Högbytorp | UNT | 0.808 | 24.1 | 3.48 | 7843 | 27500 | 1822 | 1632 | 0.47 | 0.15 | 5.50 | 86.8 | 14.8 | <d.l. | 0.94 | 18.48 |
|  | PE | 0.807 | 29.1 | 3.67 | 4899 | 29722 | 1452 | 1679 | 0.50 | 0.16 | 5.39 | 58.7 | 10.6 | <d.l. | 0.76 | 18.27 |
| INERIS Phytoagglo | PE | 0.793 | 19.5 | 3.11 | 7598 | 31176 | 1429 | 1513 | 0.22 | 0.15 | 8.52 | 96.8 | 13.6 | <d.l. | 1.57 | 27.17 |
| INERIS Phytosed | PS | 0.802 | 24.6 | 3.48 | 9547 | 35837 | 1436 | 1582 | 0.57 | 0.11 | 4.46 | 146.4 | 12.3 | <d.l. | 0.95 | 62.27 |
| Lommel | UNT | 0.793 | 21.9 | 2.68 | 16763 | 17784 | 3693 | 2284 | 19.76 | 0.21 | 6.15 | 58.7 | 41.0 | <d.l. | 0.75 | 523.04 |
|  | PE | 0.793 | 23.7 | 3.54 | 12970 | 18082 | 2362 | 2269 | 18.05 | 0.24 | 5.20 | 79.1 | 32.6 | <d.l. | 1.43 | 312.43 |
| Piekary | UNT | 0.780 | 7.0 | 1.13 | 11149 | 34534 | 3201 | 662 | 7.78 | 0.08 | 13.94 | 169.5 | 33.3 | <d.l. | 13.26 | 92.81 |
|  | PS1 | 0.830 | 17.6 | 2.14 | 10909 | 33212 | 3889 | 1286 | 9.50 | 0.14 | 8.43 | 79.7 | 24.6 | <d.l. | 10.95 | 124.85 |
|  | PS2 | 0.818 | 23.4 | 2.73 | 9381 | 19677 | 6984 | 1673 | 2.30 | 0.22 | 7.88 | 89.7 | 18.5 | <d.l. | 6.35 | 148.86 |
| Biogeco | UNT | 0.804 | 13.1 | 2.26 | 12469 | 13896 | 3063 | 738 | 0.27 | 0.13 | 11.13 | 61.4 | 28.2 | <d.l. | 0.41 | 18.47 |
|  | PE1 | 0.811 | 24.8 | 3.85 | 8047 | 13728 | 3506 | 1612 | 0.08 | 0.15 | 9.85 | 75.2 | 24.3 | <d.l. | 0.60 | 17.38 |
|  | PE2 | 0.808 | 33.7 | 4.71 | 10527 | 25866 | 2759 | 2611 | 0.19 | 0.92 | 8.01 | 57.2 | 22.2 | <d.l. | 0.61 | 23.40 |
|  | PS | 0.820 | 18.0 | 2.88 | 9313 | 14274 | 3319 | 992 | 0.08 | 0.08 | 9.13 | 59.7 | 40.1 | <d.l. | 0.64 | 25.67 |
| Touro | UNT |  |  | 0.28 | 7713 | 42820 | 10164 | 3541 | 2.65 | 60.52 | 278.82 | 21438.8 | 1655.4 | 74.48 | 4.28 | 642.81 |
|  | UNT+OM | 0.828 | 25.9 | 3.66 | 8797 | 41229 | 2769 | 2520 | 0.58 | 0.58 | 8.31 | 208.3 | 478.3 | <d.l. | 0.65 | 79.84 |
|  | PE | 0.835 | 52.4 | 5.40 | 12631 | 52514 | 2997 | 3694 | 0.39 | 0.08 | 8.93 | 105.0 | 70.7 | <d.l. | 0.97 | 62.92 |
|  | PS | 0.818 | 40.9 | 4.60 | 11086 | 48680 | 2716 | 3525 | 0.14 | 0.15 | 9.30 | 107.5 | 52.3 | <d.l. | 0.17 | 37.89 |
| Control soil | CONT | 0.818 | 37.8 | 4.75 | 10384 | 36526 | 2112 | 3002 | 0.08 | 0.22 | 4.71 | 71.7 | 19.3 | <d.l. | 0.90 | 20.29 |

Table S4: pH, electrical conductivity (EC, expressed in mS/cm2) and the soluble concentrations of macronutrients (mg L-1) in the soil pore water (SPW) samples from a pot experiment using soils from the 10 European GRO field trials. For each column the maximum (for pH, minimum) values are indicated in bold type.

| Site | Treatment | pH | EC | Macronutrients (mg L-1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ca | K | Mg | Na | P |
| Arnoldstein | UNT | 5.81 | 0.705 | 88.87 | 10.18 | 20.10 | 2.64 | 0.00 |
|  | PS | 6.34 | 0.993 | 121.00 | 13.40 | 35.03 | 3.21 | 0.00 |
| Bettwiesen | UNT | 7.53 | 0.781 | 99.87 | 44.90 | 17.77 | 0.71 | 1.18 |
|  | PE | 7.53 | 0.758 | 88.27 | 58.27 | 19.27 | 1.63 | 2.13 |
| Freiberg-Halsbrücke | UNT | 5.97 | 0.841 | 122.97 | 20.60 | 16.80 | 3.43 | 0.00 |
|  | PE | 6.19 | 0.696 | 99.97 | 19.97 | 14.43 | 2.02 | 0.00 |
| Högbytorp | UNT | 7.69 | 0.768 | 114.03 | 4.19 | 9.69 | 32.63 | 0.09 |
|  | PE | 7.61 | 2.600 | 93.20 | 161.67 | 31.03 | 331.33 | 0.22 |
| INERIS Phytoagglo | PE | 7.98 | 0.333 | 51.90 | 16.50 | 3.96 | 3.77 | 0.17 |
| INERIS Phytosed | PS | 7.78 | 0.586 | 95.03 | 23.93 | 4.63 | 1.71 | 0.00 |
| Lommel | UNT | 6.29 | 0.447 | 62.20 | 24.99 | 5.85 | 2.09 | 1.50 |
|  | PE | 6.44 | 0.485 | 58.53 | 31.17 | 6.81 | 2.84 | 0.99 |
| Piekary | UNT | 7.92 | 0.397 | 67.87 | 4.14 | 7.51 | 1.36 | 0.00 |
|  | PS1 | 8.02 | 1.131 | 209.97 | 7.63 | 54.75 | 2.47 | 0.00 |
|  | PS2 | 8.12 | 0.610 | 49.73 | 3.99 | 47.97 | 2.42 | 0.00 |
| Biogeco | UNT | 7.34 | 0.555 | 89.50 | 9.21 | 2.93 | 4.28 | 0.00 |
|  | PE1 | 7.06 | 0.567 | 196.63 | 24.63 | 36.43 | 9.58 | 0.76 |
|  | PE2 | 7.73 | 0.546 | 68.63 | 25.80 | 15.43 | 6.80 | 1.76 |
|  | PS | 7.05 | 0.319 | 43.37 | 6.89 | 6.07 | 2.73 | 0.00 |
| Touro | UNT | 3.42 | 4.563 | 376.67 | 5.37 | 171.33 | 6.87 | 0.00 |
|  | UNT+OM | 5.68 | 2.327 | 476.67 | 25.63 | 55.07 | 13.90 | 0.00 |
|  | PE | 6.92 | 2.337 | 312.20 | 27.67 | 43.80 | 7.58 | 0.60 |
|  | PS | 7.30 | 1.473 | 253.33 | 24.80 | 36.13 | 9.16 | 0.15 |
| Control soil | CON | 7.71 | 0.370 | 53.00 | 25.17 | 5.69 | 1.64 | 4.04 |

Table S5:Trace element concentrations (expressed as μg L-1) in the soil pore water (SPW) samples from a pot experiment using soils from the 10 European GRO field trials. For each column the maximum values are indicated in bold type. Empty cells indicate values not measured.

| Sites | Treatments | Trace elements (μg L-1) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | B | Co | Cu | Fe | Mn | Mo | Ni | Zn | Al | As | Cd | Cr | Pb |
| Arnoldstein | UNT | 5.03 | 0.16 | 10.02 | 17.47 | 76.93 | 0.00 | 11.09 | 3613.3 | 126.7 | 0.55 | 32.47 | 0.11 | 18.64 |
|  | PS | 0.00 | 0.35 | 12.91 | 37.37 | 31.23 | 0.00 | 6.56 | 1493.3 | 78.1 | 0.91 | 14.84 | 0.29 | 9.07 |
| Bettwiesen | UNT | 0.00 | 0.00 | 11.50 | 28.73 | 0.00 | 1.57 | 2.82 | 104.7 | 0.0 | 2.72 | 0.15 | 2.16 | 0.00 |
|  | PE | 8.60 | 0.12 | 17.33 | 37.50 | 0.00 | 3.35 | 6.03 | 144.3 | 0.0 | 4.57 | 0.22 | 4.66 | 0.00 |
| Freiberg-Halsbrücke | UNT | 29.23 | 0.11 | 11.90 | 49.27 | 46.70 | 0.00 | 4.04 | 216.3 | 371.7 | 15.30 | 4.66 | 0.55 | 2.62 |
|  | PE | 34.93 | 0.13 | 10.97 | 29.50 | 118.30 | 0.00 | 2.86 | 63.6 | 220.3 | 14.83 | 2.75 | 0.50 | 1.78 |
| Högbytorp | UNT | 64.23 | 0.00 | 0.00 | 0.00 | 0.00 | 5.66 | 2.48 | 0.0 | 0.0 | 0.96 | 0.00 | 0.23 | 0.00 |
|  | PE | 2023.33 | 1.70 | 26.43 | 171.67 | 0.00 | 16.00 | 20.67 | 4.6 | 111.2 | 1.74 | 0.06 | 3.21 | 0.28 |
| INERIS Phytoagglo | PE | 66.37 | 0.00 | 24.53 | 0.00 | 0.00 | 12.40 | 3.08 | 11.4 | 0.0 | 5.42 | 0.00 | 1.36 | 0.59 |
| INERIS Phytosed | PS | 53.00 | 0.00 | 8.61 | 0.00 | 0.00 | 16.35 | 2.11 | 111.3 | 22.9 | 1.59 | 0.29 | 0.27 | 0.00 |
| Lommel | UNT | 6.40 | 0.00 | 26.73 | 65.73 | 0.00 | 1.09 | 3.10 | 3016.7 | 314.3 | 30.60 | 22.17 | 1.14 | 16.12 |
|  | PE | 0.00 | 0.10 | 18.63 | 52.17 | 0.00 | 1.34 | 5.15 | 2146.7 | 242.7 | 31.77 | 17.20 | 1.25 | 13.43 |
| Piekary | UNT | 31.33 | 0.00 | 3.26 | 0.00 | 0.00 | 4.89 | 0.82 | 42.0 | 0.0 | 37.97 | 1.64 | 0.33 | 8.02 |
|  | PS1 | 52.90 | 0.28 | 11.33 | 40.20 | 0.00 | 4.38 | 3.06 | 202.3 | 18.6 | 17.57 | 4.73 | 0.44 | 12.21 |
|  | PS2 | 79.10 | 0.26 | 3.60 | 38.17 | 0.00 | 4.16 | 0.89 | 63.7 | 0.0 | 6.37 | 0.32 | 0.30 | 2.49 |
| Biogeco | UNT | 0.00 |  | 193.00 | 7.23 | 0.00 |  |  | 4.0 | 63.3 |  |  |  |  |
|  | PE1 | 22.77 |  | 344.23 | 41.47 | 25.93 |  |  | 40.6 | 263.5 |  |  |  |  |
|  | PE2 | 94.00 |  | 488.00 | 48.70 | 0.00 |  |  | 8.0 | 127.4 |  |  |  |  |
|  | PS | 0.00 |  | 275.00 | 25.87 | 0.00 |  |  | 0.0 | 0.0 |  |  |  |  |
| Touro | UNT | 5.33 |  | 21400 | 1160 | 67566 |  |  | 40800 | 428000 |  |  |  |  |
|  | UNT+OM | 61.70 |  | 38.50 | 40.27 | 7982.00 |  |  | 728.7 | 376.7 |  |  |  |  |
|  | PE | 32.20 |  | 158.93 | 47.30 | 37.33 |  |  | 134.3 | 275.0 |  |  |  |  |
|  | PS | 45.77 |  | 43.27 | 30.23 | 29.10 |  |  | 26.7 | 244.7 |  |  |  |  |
| Control soil | CON | 13.20 | 0.00 | 24.00 | 25.27 | 0.00 | 2.65 | 0.00 | 13.1 | 0.0 | 4.70 | 0.00 | 0.37 | 0.00 |

Table S6: Ammonium nitrate -extractable concentrations of trace elements (expressed as μg kg-1) in soils from the 10 European GRO field trials. For each column the maximum values are indicated in bold type. <lod: below detection limit. n.d. element not determined. The values of non-parametrical Spearman correlation analyses between extractable and soluble concentrations for each element are presented in the last row.

| Sites | Treatments | 1M NH4NO3-extractable concentrations (μg kg-1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cd | Cr | Cu | Ni | Pb | Zn |
| Arnoldstein | UNT | 853.3 | n.d. | 60 | n.d. | 4967 | 48467 |
|  | PS | 500.0 | n.d. | 63 | n.d. | 2613 | 23690 |
| Bettwiesen | UNT | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
|  | PE | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
| Freiberg-Halsbrücke | UNT | 216.0 | n.d. | n.d. | 41.7 | 502 | 3467 |
|  | PE | 172.7 | n.d. | n.d. | 22.0 | 407 | 1615 |
| Högbytorp | UNT | n.d. | n.d. | 60 | <lod | n.d. | <lod |
|  | PE | n.d. | n.d. | 113 | 30.0 | n.d. | 10 |
| INERIS Phytoagglo | PE | 2.1 | 9.1 | 257 | 32.7 | 32 | 2225 |
| INERIS Phytosed | PS | 15.9 | 8.4 | 105 | 29.8 | 38 | 6406 |
| Lommel | UNT | 508.0 | n.d. | n.d. | n.d. | 524 | 43483 |
|  | PE | 360.3 | n.d. | n.d. | n.d. | 389 | 27750 |
| Piekary | UNT | 366.7 | n.d. | n.d. | n.d. | 840 | 4563 |
|  | PS1 | 413.3 | n.d. | n.d. | n.d. | 803 | 7513 |
|  | PS2 | 113.3 | n.d. | n.d. | n.d. | 483 | 10547 |
| Biogeco | UNT | 1.6 | n.d. | 15000 | <lod | n.d. | 13 |
|  | PE1 | n.d. | n.d. | 3453 | <lod | n.d. | 83 |
|  | PE2 | 1.1 | n.d. | 5290 | <lod | n.d. | 43 |
|  | PS | 1.2 | n.d. | 4793 | 177.3 | n.d. | 130 |
| Touro | UNT | 7.4 | 72.3 | 29170 | 3053 | n.d. | 20033 |
|  | UNT+OM | <lod | <lod | 473 | 726.7 | n.d. | 7390 |
|  | PE | <lod | <lod | 187 | 161.3 | n.d. | 1083 |
|  | PS | <lod | <lod | 160 | 77.0 | n.d. | 180 |
| Control soil | CON | <lod | <lod | 230 | <lod | <lod | 60 |
| Correlation NH4NO3/ SPW |  | 0.087ns | n.d. | 0.716*** | 0.154ns | -0.076ns | 0.684*** |

*** p-value < 0.001

Table S 7: Performance of vegetation covers at the field trials.

| Management strategy / Sites | Plant species | Changes in labile metal(loid) pool and plant responses | References |
| --- | --- | --- | --- |
| (Aided) Phytoextraction |  |  |  |
| Bettwiesen | tobacco sunflower | After 5 years: leachate content reduced up to Zn 63%, Cd 59% and Pb 64% (plot A); decrease in metal exposure: labile pool in topsoil lowered by 45-70% for Zn, 28-67% for Cd and 26-61% for Pb. Tobacco: shoot biomass 25-37 t DW ha-1; shoot metal removals varied from 343 to 3858 g Zn ha-1 yr-1 and from 2 to 25 g Cd ha-1yr-1. Sunflower: shoot biomass 20 - 27 t DW ha-1 ; shoot metal removals reached 1498 – 5146 g Zn ha-1yr-1 and 8 - 21 g Cd ha-1yr-1 | Herzig et al (2014) |
| Freiberg-Halsbrücke | willows poplars | Total soil Cd decreased from winter 2011 to winter 2013/2014. Soil pH: under willows, it decreased between 2011 and 2012 and again after the harvest in 2013 except for two cultivars, Gudrun and Jorr; it increased under poplars after the harvest in February 2013. Under the “Tora” willow SRC, (1M NH4NO3) extractable Cd did not change after 7 years of land use change from conventional grassland to SRC. Shoot DW yields (t ha-1 yr-1) reached 15 for poplars and 14-19 for willows. Stem wood and bark ionomes (mg kg-1) of poplars and willows depended on genotypes, particularly for Cd (3-10 in wood; 8-30 in leaves). Willow cultivars Tora, Tordis and Gudrun displayed the highest wood and foliar Cd concentrations among all cultivated clones of poplars and willows | Dietzsch (2011) |
| Högbytorp | willows cv. Tora and Gudrun | After 6 years: total topsoil Cd and Pb were lower in 2 out of 4 soil treatments under the willow SRC. Gudrun and Tora did not influence total topsoil As, Cu, Hg, Co, Cr, V and Zn. Tora accumulated higher shoot Cd, Co, Mn, Pb and Zn concentrations than Gudrun in the plots treated with landfill leachates. Shoot concentrations varied in the 1-4.5 mg Cd and 40-120 mg Zn kg-1 DW ranges | Aronsson et al. (2010) |
| Ineris Creil-Phytoagglo | willows (S. viminalis), A. halleri | After 2 years; willows: survival rate 100%, increase in maximum stem height per year +500%; foliar concentrations (mg kg-1 DW): willows, Cd 4±1.5, Zn 1250±500 (bioconcentration factors: 2.5 for Cd and Zn); A. halleri: Cd 8, Zn 5 500. | Bert and Cadière (2015) |
| Lommel | willows S. schwerinii × S. viminalis cv. Tora | After 4 years: stem biomass reached 2500±2000 kg DW ha-1 yr-1; stem concentrations (mg kg-1 DW): Cd 21±1, Zn 631±72 ; phytoextraction (g ha-1 yr-1): Cd 53±42, Zn 1578±1275. After 8 years: slight decrease of the soil CEC; decrease in soil electrical conductivity; decreases in EDTA-exchangeable Cd (-26%) and Pb (-32%) concentrations, but not for Zn; no changes for (1M NH4NO3)-exchangeable Cd, Zn and Pb concentrations | Ruttens et al. (2011); Janssen (2015) |
| Biogeco | sunflower, tobacco | After 7 years. shoot biomass (t DW ha-1): sunflower 0.51 – 1.85; tobacco 1.9 – 4.1 in PE1, 1.5-4.5 in PE2; shoot Cu removal (g ha-1): sunflower 27-78 in PE1; tobacco 88-193 in PE1, 119-224 in PE2, and 128-154 in UNT. Soil (1 M NH4NO3)-extractable –Cu reduced by 65% after 6 crops. | Kolbas et al. (2011); Kolbas (2012) Kidd et al (2015) |
| Touro | tobacco | After 3 years. shoot biomass (t DW ha-1): tobacco 2.7 – 3.4; shoot Cu removal (g ha-1): tobacco 60 -135 | Kidd et al (2015) |
| In situ stabilization/ phytoexclusion |  |  |  |
| Arnoldstein | maize, barley | Use of the excluder-phenotype Bodega vs. accumulator-phenotype Hellana reduced barley grain Cd by >40 %. In combination with the incorporation of gravel sludge and red mud into the contaminated soil, a further >30 % Cd uptake could be avoided. Similar results were obtained with maize shoots (less accumulation in Falkone cultivar). In year 5, soil amendments were still effective immobilizing agents showing the efficiency and cost-effectiveness of this GRO. 1M NH4NO3-extractable Cd was reduced by >80%; Zn >90% and Pb >90%. | Friesl et al. (2006); Friesl-Hanl et al. (2009) Hua et al (2017) |
| Ineris Fresnes-sur-Escaut Phytosed | willows cv. Inger and Tordis; Deschampsia cespitosa | Deschampsia cespitosa: vegetation cover varied from 25% to 100%; no visible deficiency; common values for shoot metal(loid) concentrations (mg kg-1 DW ): Cd 0.15-0.28, Zn 150-300 Willows: Survival rate varied from 36% (Inger+ D. cespitosa) to 100% (Tordis); wood concentrations (mg kg-1DW): Cd 4 – 7.5, Zn 500-700; foliar concentrations (mg kg-1DW): Tordis (Cd 6 – 13; Zn 1000-2400) > Inger (2.5-12; Zn: 750-2100) 0.001 M Ca(NO3)2-extractable soil fraction (µg kg-1): Cd 1-2, Zn decreased from 800 to 250; no correlation with foliar concentrations of willows | Kidd et al (2015) Bert (2017, unpublished data) |
| Piekary | grass mixture with local cultivars: Festuca rubra L. cv. Atra, Poa pratensis L. cv. Alicja, Festuca arundinacea Schreb. cv. SZD, and Festuca ovina L. cv. Sima | 17 years after biosolid incorporation, water-soluble fractions of major soil contaminants (Zn, Cd, and Pb) remained at low levels, in line with soil pH and Ca-carbonate distribution over the field. Soil bacterial communities were highly diversified in amended soils. Dehydrogenases activity increased as water extractable metal (Cd, Zn) fractions in the soils were reduced. Plant cover and biomass production were highest in soils amended with biosolid combined with byproduct lime. Untreated tailings outside the reclamation area remained barren. Most persistent grass species were P. pratensis, A. capillaris and F. ovina. These species covered the largest remediated area among all grasses. A substantial part of the remediated area was covered by colonists: Calmagrostis epigejos, Hypochoeris radicata, Melandrium album, Artemisia vulgaris, Daucus carota and Solidago gigantean. | Stuczynski et al. (2007) |
| Aided phytostabilization |  |  |  |
| Biogeco | Cytisus striatus, S. caprea, Agrostis spp., D. glomerata, etc. | Agrostis castellana and A. gigantea, and C. striatus, Dactylis glomerata, S. caprea, and Holcus lanatus in a lesser extent, better survived and developed in the amended soil than the untreated soil. Across grassy species, shoot DW yields were higher for A. gigantea and D. glomerata followed by H. lanatus, A. castellana and A. capillaris. Foliar Cu concentration (mg kg-1DW) ranged from 12.5 (S. caprea) to 53 (H. lanatus) in the amended soil as compared to 18 (P. nigra) – 76 (A. gigantea) in the untreated soil. Species richness and Shannon index increased in the aided phytostabilized soil. | Bes (2008) |
| Touro | Salix spp., P. nigra, A. capillaris cv. Highland | soil NaNO3-extractable Cu concentrations remained low (<1 mg kg-1) in treated soils without influence of the vegetation cover type. In year 4, the mean height for S. viminalis was 219 cm (with plants reaching up to 390 cm), 189 cm for S. caprea (with a maximum of 340 cm), and 240 cm for P. nigra (maximum of 300 cm). Survival rate of P. nigra was 67-75% (of initial planted trees), while survival of both Salix spp. was similar but variable between sub-plots: ranging from 35 to 96% for S. viminalis and from 35 to 87% for S. caprea. Shoot DW yield ranged from 3.7 ±3.0 t ha-1 in S. caprea to 4.3 ±2.3 t ha-1 in S. viminalis. Shoot yields of P. nigra were more uniform between sub-plots with a mean biomass production of 4.3 ±0.7 t ha-1. Agrostis capillaris well established: shoot DW yield reached 2.2 t ha-1 in year 2, and increased to 4.6 t ha-1 in year 3. Leaf Cu concentrations ranged from 13.4 to 19.6 mg kg-1 with no difference between species. | Kidd et al (2015); Touceda et al (2017) |

Present address: Laboratoire Sols et Environnement, UMR 1120, INRA, Univ. Lorraine, 54518, Vandœuvre-lès-Nancy, France